# Acetate: A New Feedstock for Biomanufacturing

## Introduction

Biomanufacturing has traditionally relied on sugar-based feedstocks (e.g. glucose, sucrose) for microbial fermentation. However, these first-generation feedstocks directly compete with food and feed supply, raising concerns about food security and sustainability[1][2]. As the global demand for bio-based chemicals grows, there is increasing impetus to explore alternative carbon sources that do not divert edible resources and can leverage waste streams or CO₂. In this context, acetate (acetic acid) has emerged as a promising new feedstock for industrial biotechnology[3][4]. Acetate is a simple C₂ molecule that can be obtained from diverse non-food sources – including lignocellulosic biomass, industrial off-gases, C1 gas fermentation, and electrochemical CO₂ conversion – making it an attractive platform substrate for sustainable production of fuels and chemicals[5][6]. Figure 1 conceptually illustrates how acetate can serve as a central building block, feeding into microbial pathways (via acetyl-CoA) to yield value-added products, with the help of metabolic engineering and renewable feedstock sourcing.

Figure 1. Concept of utilizing acetate as a feedstock in biomanufacturing. Renewable or waste sources (biomass, C1 gases, etc.) are converted to a C₂ feedstock (acetate), which is assimilated by microbes into the central metabolite acetyl-CoA. Through engineered pathways, acetyl-CoA is transformed into a variety of bioproducts. This two-step “electro‐bio” approach (inorganic conversion of CO₂ to acetate, followed by biological conversion of acetate to product) enables carbon fixation into chemicals and materials, decoupling production from food-based sugars.[7][8]

Several features make acetate particularly compelling as a feedstock. Abundant and low-cost supply: Acetic acid can be generated as a byproduct of biomass pretreatment (deacetylation of hemicellulose releases acetate) and in various fermentations (overflow metabolism)[9]. It can also be produced by acetogenic bacteria from synthesis gas (CO/H₂/CO₂) or via microbial electrosynthesis, as well as by electrochemical reduction of CO₂ using renewable electricity[10][11]. These routes mean acetate can be sourced from agricultural residues, industrial off-gases, or even directly from captured CO₂, bypassing the need for edible sugars[6][11]. As a result, acetate avoids the “food vs fuel” dilemma and aligns with carbon recycling and carbon-neutral production goals. Moreover, the market price of bulk acetate (acetic acid) is often lower than that of refined sugar. For instance, recent estimates put industrial acetic acid at roughly \$300–350 per ton, versus \$500–550 per ton for glucose[12]. This economic advantage, coupled with the diversified supply, makes acetate an economically viable feedstock option[13].

Metabolic efficiency: In microbial metabolism, acetate feeds in at the level of acetyl coenzyme A (acetyl-CoA) – a central node in the tricarboxylic acid (TCA) cycle and precursor for many biosynthetic pathways. Unlike hexose sugars, which must be broken down (losing carbon as CO₂) to generate acetyl-CoA, each molecule of acetate can be directly assimilated into one molecule of acetyl-CoA with no carbon loss. This theoretical 100% carbon conversion to acetyl-CoA (versus only ~67% from glucose due to two CO₂ lost in glycolysis) means higher carbon efficiency for producing acetyl-CoA-derived compounds[14][15]. In practical terms, using C₂ feedstocks can reduce feedstock requirements by ~50% when targeting acetyl-CoA derivatives[14]. Additionally, acetate metabolism involves shorter pathways to central metabolites, potentially minimizing futile fluxes. These factors suggest that acetate could enable higher yields in processes focused on acetyl-CoA derived products (e.g. fatty acids, isoprenoids, polymers)[16][17]. Indeed, a recent analysis highlighted that for products originating from acetyl-CoA, C₂ substrates can achieve comparable yields to sugars[17].

At the same time, unique challenges accompany the use of acetate as a feedstock. Acetate is a more oxidized carbon source than sugars and yields less cellular energy upon assimilation. Fermentative growth on acetate typically produces only about 7 ATP per mole (via oxidative metabolism and substrate-level phosphorylation), compared to roughly 26 ATP per mole of glucose[18]. This lower ATP yield and the need for additional reducing power (NADPH) to assimilate acetate (which has a higher oxidation state) can limit biomass growth rates and product formation if not addressed. Furthermore, acetic acid is inhibitory at high concentrations, especially in its protonated form at low pH – it can diffuse into cells and uncouple metabolism. Many conventional production strains (e.g. E. coli lab strains or yeast) have only moderate tolerance to acetate and may exhibit slow growth or stress responses when using it as the sole carbon source[18]. These issues underscore the importance of metabolic engineering and strain development to fully realize acetate’s potential. In the following sections, we review the current state of acetate-based biomanufacturing, from the pathways enabling acetate assimilation to the latest engineering advances that overcome limitations. We compare strategies across different microorganisms and products, highlight successful case studies, and critically examine the remaining hurdles. We also discuss how acetate feedstock ties into the broader context of sustainable bioproduction (including CO₂ utilization), and we conclude with a forward-looking perspective on where this field is heading in the next 5–10 years.

## Renewable Sources of Acetate for Biomanufacturing

A key appeal of acetate is that it can be produced from a variety of renewable or waste resources, making it a flexible platform feedstock. Here we summarize the primary routes for sustainable acetate supply and their relevance to industrial biomanufacturing:

- Lignocellulosic Biomass and Agricultural Waste: Lignocellulose (e.g. corn stover, wood, grasses) contains acetylated hemicellulose; during pretreatment (especially dilute acid or steam explosion), acetyl groups are released as acetic acid. Rather than treating this acetate as a fermentation inhibitor to be removed, it can be recovered and used as feedstock. For example, pyrolysis oil from biomass contains a significant fraction of acetic acid, which can be isolated and purified[19]. Waste streams like pulp and paper effluents or food waste fermentations also often contain acetate. Utilizing these biomass-derived acetates in fermentation closes the loop on waste carbon and adds value to agricultural residues[20]. Recently, Kiefer et al. demonstrated high-cell-density cultivation of Corynebacterium glutamicum on lignocellulosic acetate (acetate derived from wood hydrolysate), achieving efficient growth by controlling pH and feeding rate[21][22]. This exemplifies the viability of biomass-to-acetate-to-product processes.

- Syngas Fermentation (Gas Fermentation): Certain anaerobic microbes (acetogens) naturally ferment synthesis gas (CO + H₂ + CO₂ mixtures) into acetic acid as their primary product via the Wood-Ljungdahl pathway. Industrial gas fermentation platforms (e.g. using Clostridium autoethanogenum or Acetobacterium sp.) can convert waste gases from steel mills or syngas from biomass gasification into acetate at scale[18]. This acetate can either be the final product or an intermediate that is fed to a second microbe. Gas fermentation to acetate is attractive because it fixes CO₂ or CO with hydrogen, effectively turning waste gases into a liquid carbon feedstock. Companies are exploring two-step systems where an acetogen first produces acetate from CO-rich off-gas, and a production strain then converts the acetate to chemicals or fuels, taking advantage of the higher yield and robustness of the second stage organism. This approach leverages the best of both worlds: gas fermenters excel at carbon fixation but have limited product scope, whereas model organisms (like E. coli or yeast) can be engineered for a wide product range but prefer liquid substrates[23][11]. Acetate serves as the link between them.

- Electrochemical CO₂ Reduction to Acetate: One of the most exciting developments enabling acetate feedstock is the electrochemical production of acetic acid from CO₂. Advanced catalysts (typically based on nanostructured copper) can reduce CO₂ in an electrolysis cell to C₂ products, predominantly acetate, at high selectivity and increasing yields[10][24]. For instance, a recent study achieved >50% carbon efficiency to acetate using a tandem electrocatalysis: CO₂ is first reduced to CO (using a Ag catalyst), then CO is further reduced to acetate on Cu, in a two-step electrolyzer system[24][25]. The acetate is collected in the aqueous electrolyte. Remarkably, this system produced a concentrated acetate stream that could be fed directly to microbes without toxic effects[10][26]. Hann et al. (2022) demonstrated a “artificial photosynthesis” setup where solar-powered CO₂ electrolysis generated acetate, which was then used as the sole carbon and energy source to grow organisms including Saccharomyces cerevisiae (yeast), Chlamydomonas (alga), and even crop plant tissues, all in the dark[10][27]. This proof-of-concept illustrates how acetate can enable food or biomaterial production completely independent of natural photosynthesis – at dramatically higher solar-to-biomass efficiency than agriculture[26]. In the chemical realm, Lee et al. (2025) recently reported a two-step CO₂-to-PHB (bioplastic) process: electrocatalysis converted CO₂ to acetate, and an engineered bacterium (Cupriavidus necator) fed on the acetate to produce poly(3-hydroxybutyrate)[28][29]. Such hybrid electrochemical–biological approaches are expected to grow, positioning acetate as a key intermediary for CO₂ valorization in the coming decade.

- Other Routes: Acetate can also be produced by oxidative fermentation of ethanol (e.g. using Acetobacter species to make vinegar). While ethanol itself is often produced from sugar or syngas, in some contexts an ethanol-to-acetate step could be integrated if it fits a process need. Additionally, methane oxidation (e.g. via methane-utilizing bacteria or engineered pathways) can yield acetate as an intermediate, although this is less common industrially. Overall, any process that yields a surplus of acetate (such as certain amino acid fermentations where acetate is a byproduct) could funnel that acetate into further bioproduction instead of treating it as waste[9][30]. This highlights the concept of circular biorefineries where one process’s waste acetate becomes another process’s feed.

In summary, acetate is uniquely situated at the nexus of renewable energy and biotechnology. It can be sourced from non-petrochemical, non-food inputs – from woody biomass to greenhouse gases – making it a platform feedstock for sustainable biomanufacturing. The diversity of production routes also provides supply security and potential for regional sourcing (for example, biomass in agricultural regions, CO₂-to-acetate where cheap electricity is available). These advantages motivate the development of microbial processes that can efficiently convert acetate into desired products, which we discuss next.

## Microbial Assimilation of Acetate: Pathways and Engineering

To harness acetate as a feedstock, microbes must efficiently take up and assimilate it into their central metabolism. Acetate utilization in most organisms proceeds through conversion to acetyl-CoA, a pivotal metabolite. There are two principal pathways for this conversion in bacteria (exemplified by E. coli): (1) the acetyl-CoA synthetase (ACS) pathway, and (2) the acetate kinase-phosphotransacetylase (AckA–Pta) pathway[31]. In the ATP-dependent ACS pathway, acetate is activated to acetyl-CoA in a single step by acetyl-CoA synthetase (Acs), consuming 1 ATP per acetate. The AckA–Pta route is a two-step, lower-energy pathway: acetate is first phosphorylated by AckA to acetyl-phosphate, then Pta transfers the acetyl group to CoA, yielding acetyl-CoA[32]. Many bacteria possess both routes. Notably, the ACS pathway is essentially irreversible (driven by ATP hydrolysis and pyrophosphate removal) and has a high affinity for acetate (Kₘ ~0.2 mM), whereas the AckA–Pta pathway is reversible and has a much higher Kₘ (~7 mM for AckA)[33][34]. Consequently, Acs is crucial for scavenging low concentrations of acetate and driving assimilation forward, while the AckA–Pta enzymes are often more active during overflow metabolism (when cells excrete acetate under glucose excess) running in reverse[35][34].

Engineered strains have taken advantage of these differences. Overexpressing acs in E. coli significantly improves growth on acetate, enabling faster uptake and acetyl-CoA generation[34]. In contrast, overexpressing ackA and pta can be counterproductive for growth, likely because the reversible pathway may siphon acetyl-CoA back to acetate or otherwise mis-regulate the flux[34][36]. Many acetate assimilation strategies therefore focus on boosting the Acs route (or equivalent high-affinity pathways) and minimizing acetate dissimilation. Some E. coli strains naturally excel at acetate utilization – notably strain W (ATCC 9637), an old soil isolate. E. coli W has long been observed to grow robustly on acetate and tolerate acidic conditions better than the standard K-12 strains[37]. Genomic analysis revealed that W carries unique alleles and possibly regulatory differences that enhance stress resistance[38][39]. Indeed, recent studies used E. coli W as a host for acetate-based production of chemicals like isopropanol and itaconic acid, achieving higher titers than comparable K-12 strains[40][41]. This suggests that choosing or evolving strains inherently adept at acetate metabolism can simplify bioprocess development.

Once converted to acetyl-CoA, acetate carbon can enter central pathways. Aerobic growth on acetate typically relies on the TCA cycle plus the glyoxylate shunt. The glyoxylate cycle (via isocitrate lyase aceA and malate synthase aceB) bypasses the CO₂-generating steps of the TCA cycle, allowing cells to assimilate two acetyl-CoA molecules into four-carbon intermediates without losing carbon as CO₂[42][43]. This is essential for growth on acetate because it permits net production of oxaloacetate (and hence gluconeogenic precursors) from acetyl-CoA. However, routing flux through the glyoxylate shunt instead of the full TCA cycle comes at the cost of energy generation – fewer NADH and ATP are produced. This trade-off partly explains why acetate supports slower growth: cells prioritize carbon conservation (anabolism) over energy yield. Metabolic engineers have attempted to optimize the glyoxylate bypass flux to improve acetate utilization. For example, overexpression of isocitrate lyase or engineering its regulation can increase acetate assimilation, but excessive flux can also cause imbalances (e.g. draining acetyl-CoA from energy-generating TCA)[44][43]. Fine-tuning is required to maximize growth or production from acetate.

Another challenge is that not all target products are direct derivatives of acetyl-CoA. Many high-value compounds (e.g. aromatics, certain amino acids) require upstream glycolytic or pentose phosphate intermediates (like phosphoenolpyruvate or erythrose-4-phosphate) as precursors. When using acetate, which enters metabolism downstream, cells may become starved for these upstream precursors. To address this, metabolic pathways must be replenished (“anaplerotic” routes) to drive carbon from acetyl-CoA back towards gluconeogenesis. Several strategies have been demonstrated (see Figure 2 for a pathway schematic). One approach is expressing pyruvate-ferrodoxin oxidoreductase (PFOR, e.g. ydbK gene in E. coli), which can reversibly convert acetyl-CoA and CO₂ into pyruvate[45][46]. Overexpressing ydbK effectively fixes carbon (consumes CO₂) and was shown to improve growth and production of non–acetyl-CoA products from acetate[45]. Another strategy is enhancing PEP carboxykinase (pckA) or malic enzyme (maeB), to boost the flow from TCA cycle intermediates to PEP and pyruvate[47][48]. In one study, co-overexpression of pckA and maeB in acetate-grown E. coli increased availability of gluconeogenic precursors, though interestingly it did not outperform PFOR (ydbK) alone[47][49]. The unique advantage of PFOR is that it fixes CO₂ instead of releasing it, making it inherently more carbon-efficient than PEP carboxykinase or malic enzyme (which produce CO₂). For this reason, there is interest in transplanting or engineering alternative carbon fixation enzymes into acetate-utilizing hosts to improve carbon yield.

Figure 2. Metabolic pathways for acetate assimilation and conversion to products in an engineered E. coli (based on pathways summarized by Gu et al., 2024[50]). Acetate is taken up and converted to acetyl-CoA either by acetate kinase/phosphotransacetylase (ackA, pta) forming acetyl-phosphate (AcP), or by acetyl-CoA synthetase (acs) with ATP. Acetyl-CoA enters the TCA cycle (red) or is assimilated via the glyoxylate shunt (aceA, aceB) to yield oxaloacetate and malate for gluconeogenesis. Key engineered routes to improve acetate utilization are shown: (1) Gluconeogenic bypasses (blue arrows) – e.g. pckA to convert oxaloacetate to PEP, maeB to convert malate to pyruvate, or ydbK (pyruvate synthase) to directly carboxylate acetyl-CoA to pyruvate (fixing CO₂). These help generate sugar-phosphate precursors for products like amino acids or aromatics. (2) NADPH supply – isocitrate dehydrogenase (ICDH) and other steps (not shown) must balance flux to provide reducing power for biosynthesis. (3) Product pathways (color-coded): e.g. isobutanol (green, via the Ehrlich pathway from ketoisovalerate), 2,3-butanediol (yellow, via acetolactate pathway), mevalonate/isoprenoids (cyan, “MVA pathway” from acetyl-CoA), itaconic acid (pink, via cad converting cis-aconitate), polyhydroxybutyrate (PHB) and 4HB polymer (brown and light blue, via acetyl-CoA to acetoacetyl-CoA to 3HB-CoA/4HB-CoA). Enzymes and relevant genes are indicated next to arrows. By integrating such pathways, E. coli can produce a wide range of compounds from acetate, though careful tuning of fluxes (ATP and NADPH use, glyoxylate cycle activity, etc.) is required for high efficiency.[51][52]

Beyond pathway modifications, global metabolic engineering is often needed to optimize acetate utilization. One recurring issue is the buildup of excess ATP in acetate-grown cells – paradoxically, while acetate yields less net ATP, E. coli can accumulate a surplus of ATP if acetyl-phosphate pools get too high, which in turn can dysregulate metabolism and slow growth[18]. To counter this, Seong et al. (2020) employed adaptive laboratory evolution to derive E. coli mutants that grow faster on acetate by increasing their maintenance ATP consumption[53][54]. Essentially, the evolved strain “burns off” extra ATP, relieving feedback inhibition and allowing higher acetate flux. Another approach to improve acetate tolerance is altering global regulators. For instance, CRP (cAMP receptor protein), a key regulator of carbon catabolism, has been “rewired” to enhance acetate usage: by mutating CRP or its binding sites, researchers have created E. coli strains that no longer strongly prefer glucose and can more readily metabolize acetate even in rich media[55][56]. Similarly, overexpression of certain sigma factors or stress response regulators can increase acid tolerance, allowing cells to withstand higher acetate concentrations. These systems-level tweaks complement pathway engineering to create robust acetate-utilizing biocatalysts.

It is worth noting that yeast and other eukaryotic microbes also metabolize acetate, though there are differences. Saccharomyces cerevisiae assimilates acetate via an acetyl-CoA synthetase (Acs1/Acs2) in the peroxisome and mitochondria, and requires the glyoxylate cycle (which in yeast is active in the peroxisome) for growth on C₂ substrates. Engineered yeast have been used for acetate-based production of chemicals (e.g. itaconic acid and malic acid in Yarrowia lipolytica[57][58]). Generally, the same principles apply: boosting acetate uptake (e.g. overexpressing ACS, engineering transporters), ensuring anaplerotic flux, and mitigating acetate stress are key. Some oleaginous yeasts natively consume acetate and accumulate lipids, showing that eukaryotes can be effective acetate bioconversion platforms[59][60]. However, slow growth and byproduct formation (like ethanol or acetate excretion under certain conditions) can be issues to overcome.

In summary, microbes have the metabolic capability to use acetate but benefit greatly from metabolic engineering for industrial performance. Strategies include: reinforcing high-affinity acetyl-CoA synthesis (ACS pathway), adjusting the glyoxylate vs TCA flux balance, introducing carbon fixation or other bypasses to supply upstream precursors, enhancing NADPH generation for biosynthesis, and globally adapting the cell to acetate as a preferred substrate. Over the past decade, these interventions have enabled a series of successful demonstrations of acetate-driven bioproduction, as discussed in the next section.

## Biomanufacturing Case Studies: Products from Acetate

Thanks to the developments in strain engineering, an expanding array of chemicals have now been produced from acetate as the sole or primary carbon source. Table 1 highlights selected examples spanning fuels, organic acids, amino acids, polymers, and specialty chemicals. These case studies underscore both the potential and the current limitations of acetate-fed bioprocesses.

Table 1. Representative bioproducts synthesized from acetate feedstocks. (Yields are given in mol product per mol acetate when available, illustrating carbon efficiency. All processes used engineered microbes under aerobic conditions, with acetate as the sole carbon source, unless noted otherwise.)

| Host organism | Product | Titer (g/L) | Yield (mol/mol) | Reference |
| --- | --- | --- | --- | --- |
| E. coli W3110 | L-Threonine (amino acid) | 45.8 | 0.22 | Vo et al., 2024[61] |
| E. coli W | Isopropanol (biofuel) | 13.3 | 0.196 | Kutscha et al., 2024[62][41] |
| C. glutamicum | Itaconic acid (C₅ acid) | 5.0 | 0.116 | Schmollack et al., 2022[63] |
| E. coli W | GABA (4-aminobutyric acid) | 2.54 | 0.342 | Beck et al., 2024[64] |
| C. necator (H16) | PHB biopolymer (polyester) | 43 | – | Garcia-Gonzalez et al., 2018[65][66] |
| E. coli BL21(DE3) | Naringenin (flavonoid) | 0.122 | – | Kim et al., 2024[67] |

(PHB = poly-3-hydroxybutyrate; GABA = γ-aminobutyric acid. Yields marked "–" were not reported or not applicable due to polymer product.)

These examples demonstrate that acetate can support high-level production of certain compounds, approaching titers seen with sugar feedstocks. For instance, the threonine titer of 45.8 g/L achieved by Vo et al.[61] is on par with commercial fermentation titers on glucose. This was accomplished by a modular engineering approach in E. coli W3110, involving overexpression of key threonine pathway enzymes and optimization of carbon flux from acetate to the aspartate family amino acid pathway[61]. The relatively high yield (0.22 mol/mol, ~44% of theoretical maximum) indicates effective carbon utilization. Another notable success is the production of isopropanol (an important solvent and fuel) at over 13 g/L[62]. Kutscha et al. optimized an E. coli W strain for acetate-based isopropanol synthesis, using a two-stage process (growth phase then production phase) to achieve the highest reported isopropanol titer from acetate to date[68]. Metabolic analysis revealed that ATP demand and NADPH balance were limiting factors; the study identified that enhancing isocitrate dehydrogenase flux (for NADPH) and moderating the glyoxylate cycle were crucial for driving carbon to the product[69][70]. This underlines how system-level understanding is needed to push yields higher.

Organic acids like itaconic acid (a platform chemical and acrylic precursor) have also been produced from acetate in different hosts. C. glutamicum, a gram-positive bacterium known for tolerance and diverse feedstock use, was engineered to produce itaconate from acetate with a titer around 5 g/L[63]. Although yield was modest (~0.11 mol/mol), this result is significant as it demonstrates a non-E. coli chassis effectively converting acetate to a non-native product. In parallel, E. coli itself has been used for itaconate production from acetate – Noh et al. (2018) employed an acid-tolerant E. coli W strain to produce ~3.6 g/L itaconic acid in a fed-batch process[71][72]. They noted that E. coli W’s innate robustness reduced the need for extensive acid stress engineering. Together, these studies suggest that acidic products (like itaconate, succinate, etc.) can be made from acetate, but controlling pH and improving export of the product will be key to reaching higher titers.

Another compelling case is biopolymers: Polyhydroxyalkanoates (PHAs such as PHB) are biodegradable polyesters traditionally made by feeding sugars or lipids to bacteria like Cupriavidus necator. Using acetate as carbon source for PHB has been explored to connect CO₂ utilization with bioplastic production[65]. Garcia-Gonzalez et al. compared PHB production from CO₂ (via autotrophic fixation) versus from supplied acetate in C. necator, finding that the acetate route achieved substantially higher productivity[65][66]. In recent work, Sun et al. (2020) directly compared glucose, ethanol, and acetate for PHB production in E. coli, observing that ethanol and acetate outperformed glucose in PHB yield (ethanol giving the highest yield, 0.935 g PHB per g, due to no carbon loss, followed by acetate, while glucose was lowest)[73][59]. This highlights an important point: for products that are themselves acetyl-CoA derivatives, using a C₂ feedstock can avoid the inherent carbon loss of glycolysis, resulting in better yields. However, ethanol often provides more reducing power than acetate (ethanol oxidation to acetyl-CoA yields NADH), which can further enhance yields as seen in that study[73]. Thus, one controversy in the field is whether acetate or other unconventional feeds (like ethanol, methanol, formate) are “best” for a given process. Ethanol, for example, has advantages in energy yield and is also not a food sugar, but its production usually starts from sugars or syngas via additional steps[59]. Acetate may be more directly accessible from CO₂ and waste, tipping the balance in its favor for long-term sustainability despite the extra reducing power requirement. These trade-offs are the subject of ongoing research and techno-economic analysis.

On the specialty chemical front, the production of complex molecules from acetate has been demonstrated but remains challenging. A striking example is the plant flavonoid naringenin (an antioxidant of interest for supplements and as a chemical precursor). Kim et al. (2024) engineered E. coli to produce 122 mg/L of naringenin from acetate – the first report of a polyphenolic natural product made from a non-sugar feedstock[74][67]. While the titer is low, it proves that even multistep biosynthetic pathways (involving malonyl-CoA, which comes from acetyl-CoA) can be supported by acetate metabolism. The bottlenecks in such cases often include insufficient precursor supply (malonyl-CoA, in naringenin’s case) and high cofactor demands, as well as pathway enzyme limitations. The authors alleviated some constraints by optimizing flux at a key node (acetyl-CoA to malonyl-CoA) via transcriptional and translational control, improving naringenin titer[61][75]. This kind of flux optimization will be essential when pushing acetate-fed systems toward high-value, complex products.

We also note the successful fermentative production of amino acids like threonine (Table 1) and specialty monomers like 4-aminobutyrate (GABA) from acetate. The GABA production by Beck et al. (2024) achieved a yield of 0.34 mol/mol, which is quite high in carbon efficiency[64]. Their approach used “stepwise flux optimization”, likely tuning the expression of enzymes in the succinate-semialdehyde pathway and balancing the upstream supply from acetyl-CoA. Such results are promising for commercial amino acid fermentation using acetate – a process which could avoid using food sugars and might integrate with CO₂ capture (as GABA or other amino acids could be made from industrial off-gas via acetate).

Overall, these case studies collectively show that acetate can be converted into a broad spectrum of products, from bulk commodities to fine chemicals. However, a general trend is that titers and yields are currently lower on acetate than the best processes on glucose, especially for products requiring complex precursor balancing or many enzymatic steps. Many of the above examples are proof-of-concept scale (shake flasks or small bioreactors). Only a few (like threonine, isopropanol) have reached tens of grams per liter. This is an active area of scale-up research. Encouragingly, the yields in terms of carbon efficiency are often decent (20–40% of theoretical) even in first demonstrations, leaving room for further improvement. With continued strain engineering – improving tolerance, expression of pathways, and cofactor regeneration – and with bioprocess optimization (e.g. fed-batch feeding strategies, two-stage fermentations, in situ product removal for inhibitory products), we expect the performance gap between acetate and traditional feedstocks to continue narrowing.

## Challenges and Knowledge Gaps

Despite the progress, several critical challenges remain before acetate can be widely adopted as a feedstock in industrial biomanufacturing. Below we discuss the main technical and strategic hurdles, as well as areas of uncertainty and active investigation.

- Lower Growth Rates and Productivity: Microbes typically grow slower on acetate than on sugars, which can limit volumetric productivity (product formed per unit time). This is fundamentally due to the lower ATP yield from acetate catabolism and the need for gluconeogenesis. Even with engineering, E. coli on acetate may have a maximum specific growth rate around 0.3–0.4 h⁻¹, versus >0.6 h⁻¹ on glucose in similar conditions. Slower growth means longer fermentation times to reach high cell density or product titer. Some strategies to overcome this include fed-batch processes that optimize biomass formation in one phase and production in another (as done for isopropanol[68]), or using growth-coupled production designs where acetate consumption is tied to product synthesis, forcing the cells to convert acetate efficiently as they grow. Adaptive evolution, as mentioned, can also yield faster-growing acetate strains[53]. However, a fundamental challenge is that any obligate aerobic use of acetate will yield less energy; thus, novel approaches such as supplying alternative energy sources (e.g. a small amount of hydrogen gas, or using co-substrates like formate for extra reducing power) are being considered to boost acetate metabolism. This mixotrophic concept – using acetate for carbon and another substrate for energy – could potentially improve productivity but adds process complexity.

- Acetate Toxicity and Inhibition: Acetic acid is well known as a growth inhibitor (even when not used as feedstock, it’s a common byproduct that inhibits fermentations). At low pH, undissociated acetic acid enters cells and disrupts pH balance and metabolism. Even at neutral pH (where acetate is dissociated), high acetate concentrations (>5–10 g/L) can impose osmotic and stress responses. Many of the studies above kept acetate concentrations in the range of 3–30 mM (0.2–1.8 g/L) in fed-batch to mitigate toxicity[40][41]. For industrial relevance, processes will need to tolerate higher concentrations to reduce reactor volume and avoid diluting the feed. Possible solutions include strain tolerance engineering (e.g. via global regulators or efflux pumps), and pH management strategies (feeding acetate as a salt at neutral pH, or using bioreactors with continuous removal of acetic acid). E. coli W’s better acetate tolerance is a clue that there are genetic factors worth exploring – for example, differences in its cell membrane or stress response regulation[38]. Other organisms like Acetobacter or Pseudomonas are naturally very acetate-tolerant and could be mined for tolerance genes (such as acetate exporters or acid shock proteins). Biofilm-based systems or cell immobilization might also help, as they sometimes confer greater stability in harsh conditions. Overall, improving tolerance is a key knowledge gap – we need more understanding of how chronic acetate exposure affects cell physiology and which cellular systems (membrane, DNA, etc.) are most vulnerable.

- Cofactor Imbalance (NADPH/NADH): When using acetate, obtaining enough reducing power for biosynthesis is a concern. Acetate assimilation via the glyoxylate cycle produces less NADH than glucose oxidation would, and it produces no NADPH directly. Yet many product pathways (especially for reduced products like alcohols or fatty acids) require substantial NADPH. If the cell’s internal NADPH supply (e.g. from isocitrate dehydrogenase or pentose phosphate pathway operating in reverse) is insufficient, it can become the bottleneck. This can lead to byproducts or stalled pathways. Some studies have noted that the isopropanol yield was limited by NADPH availability[69]. Potential fixes are overexpressing transhydrogenases (to convert NADH to NADPH), or engineering alternative NADPH generation routes (like using a NADP-dependent malic enzyme or an NADP-dependent acetyl-CoA dehydrogenase). Synthetic biology approaches might also introduce non-native carbon fixation cycles that yield NADPH while assimilating acetate. At present, the quantitative contribution of each NADPH source during growth on acetate is not fully mapped – more systems biology (omics and flux analysis) is needed to identify how to optimally reroute flux for balanced cofactor use.

- Carbon Yield and Byproducts: Achieving high carbon yield from acetate (i.e. most of the acetate ends up in the product) remains challenging for products that require diversion of carbon to cell mass or to CO₂. While acetyl-CoA derived products can, in theory, approach ~100% carbon yield (since all carbon goes through acetyl-CoA), in practice cells need to burn some acetate for energy (oxidizing it to CO₂). For example, the threonine process above obtained 35% of theoretical yield[76], meaning much of the acetate was still used for cell growth or respired. One way to improve yield is to decouple growth from production – e.g. grow cells on acetate to a certain biomass, then switch conditions so that incoming acetate is channeled mostly to product (perhaps by limiting nitrogen or oxygen to slow growth as was done in isopropanol production[68]). Another approach is to supply an external energy source (like a low voltage current or hydrogen gas in a bioreactor) to partially power cell maintenance, allowing more acetate carbon to go to product. This concept of electrically enhanced fermentation is very nascent, but researchers are exploring whether a cathode in a fermentor could provide reducing equivalents to organisms (essentially a hybrid of microbial electrosynthesis and fermentation)[77][78]. If successful, it could boost yields by fulfilling energy needs electrically while carbon is conserved in product. Minimizing side-product formation (like acetate being inadvertently re-produced by the cells, or formation of ethanol, lactate, etc.) is another aspect – this often requires knocking out competing pathways (as done by Seong et al. who deleted byproduct pathways in E. coli[53]) and carefully controlling oxygen supply and feed rates.

- Process Integration and Economics: From a process engineering perspective, using acetate introduces some new considerations. If acetate is produced externally (e.g. via CO₂ electrolysis or gas fermentation), the integration of that step with the fermentation step needs careful design. For instance, electrolyzers producing acetate will generate it in an electrolyte solution along with significant alkali (e.g. KOH, if K⁺ is the cation). The resulting acetate broth may have high ionic strength or impurities that can inhibit microbes[79]. In the Nature Food demonstration, the team adjusted the electrolyzer output to be directly suitable for microbes, but this may require custom configurations[24][25]. In an industrial setting, one might need to filter or preprocess the acetate solution (to remove catalysts, adjust pH/salinity) before feeding bioreactors. Each extra step adds cost. Techno-economic analyses (TEA) have begun to evaluate these systems. Early indications are that if renewable electricity is cheap and carbon is priced (for emissions), the two-step CO₂-to-acetate-to-product route can be competitive, especially for products with high market value or those made inefficiently from sugar[68][80]. However, if the target is a bulk commodity (like ethanol or PHB), the overall energy efficiency and capital cost need to be favorable. Improving electrocatalyst efficiency for acetate and achieving high acetate concentrations (>100 g/L) in the electrolyzer would make downstream fermentation much easier (less water to deal with). Thus, advancements on the inorganic chemistry side (electrochemical CO₂ reduction) will directly influence the viability of acetate biomanufacturing. This is a multidisciplinary gap – biotechnologists and electrochemists need to work together to optimize the interface between the systems.

- Comparison with Direct Gas Fermentation: A strategic debate in the field is whether to adopt a two-step process (CO₂ → acetate → product) or try to engineer single-step gas fermentation to product (CO₂ → product in one organism). Some argue that each conversion step adds inefficiency, so an organism that can take CO₂ or CO directly to the target (like acetogens engineered to produce fuels) would be superior. Indeed, gas fermentation avoids the need to handle liquid acetate altogether. However, direct gas fermentation suffers from mass transfer limitations (gases don’t dissolve well, limiting rates) and is currently limited to a narrower range of organisms and products[81][82]. The two-step strategy allows specialization: an efficient CO₂-to-acetate system coupled with an efficient acetate-to-product system[83][84]. This modularity can be more robust and easier to optimize than a single organism that must do everything (fix CO₂ and produce complex products). Still, developing a stable, low-cost acetogen + partner organism co-culture is non-trivial. There are also hybrid approaches, like feeding CO₂ to a photosynthetic microorganism that excretes acetate which is then consumed by a production strain (a concept of “microbial cooperation”). At present, the two-step approach using acetate is gaining traction, but further head-to-head comparisons (in terms of energy efficiency, cost, and scalability) between direct gas fermentation and acetate-mediated fermentation are needed. This will inform where acetate is the preferred intermediate and where it might not be necessary.

- Regulatory and Purity Considerations: If acetate is derived from waste sources, its purity can vary. Fermentation microbes are sensitive to impurities like heavy metals, sulfide, or other organics that might come along with industrial off-gas or biomass hydrolysate. Ensuring a consistent, clean acetate feedstock is important for reproducible bioprocesses. This might require new analytical and control strategies (e.g. real-time monitoring of feed composition) and possibly robust mixed cultures that can co-consume impurities. For specialty products like food or pharma ingredients made from acetate, regulatory approval will hinge on demonstrating that no harmful contaminants from the waste-derived acetate make it into the product. These are manageable issues but call for more research into feedstock pretreatment and microbial tolerance to complex feed streams.

In summary, the shift to acetate feedstocks brings to light several scientific and engineering gaps: improving microbial physiology on acetate (rate, yield, tolerance), integrating novel energy inputs, and designing end-to-end processes that are economically sound. Many of these challenges are surmountable with targeted R&D and will likely be a focus of the next wave of studies.

## Future Outlook and Perspectives (Next 5–10 Years)

Looking ahead, the use of acetate as a carbon feedstock is poised to expand and mature, driven by the imperatives of sustainability and technological innovation. Here we outline expected developments and our perspective on the field’s trajectory in the coming decade:

- Toward Commercialization: We anticipate that one or more pilot-scale acetate-to-product processes will be demonstrated, potentially at the tens-of-thousands of liters scale, within the next 5–10 years. Early candidates might be products where using acetate gives a clear advantage. For example, organic acids (like succinate, malate) or certain amino acids could be produced from acetate derived cheaply from biomass hydrolysate, thereby lowering substrate costs and avoiding raw sugar price volatility. Amino acid fermentation companies are already exploring acetate or ethanol feeds for production to reduce reliance on food sugars[4][6]. Similarly, bioplastics (PHA, PHB) might see pilot production using CO₂-electrolysis to acetate as the carbon source, tapping into the growing need for carbon-neutral materials. Success in these arenas will hinge on continued strain improvements (for titer and yield) and demonstrated integration of the acetate generation step.

- Strain Engineering Breakthroughs: On the synthetic biology front, we expect more sophisticated engineering of acetate-assimilation pathways. This could include synthetic pathways that bypass some limitations of the native routes. For instance, researchers are investigating artificial cycles or enzyme combinations that assimilate C₂ units with no carbon loss at all (beyond the glyoxylate cycle). One concept is to condense two acetate molecules into four-carbon compounds without releasing CO₂ – akin to a reverse glyoxylate or malate cycle. While no naturally carbon-conserving cycle (aside from glyoxylate) exists for acetate, advances in enzyme engineering may allow designing such pathways. Another area is protein and enzyme engineering to improve the kinetics of acetate-utilizing enzymes. The affinity of acetate kinase (AckA) might be enhanced or made irreversible, or a new enzyme could be evolved that directly converts acetate to acetyl-phosphate more efficiently. Already, progress has been made: researchers identified a phosphoketolase variant that can convert glycolaldehyde to acetyl-phosphate in one step, greatly shortening the route for ethylene glycol assimilation[85][86] – analogous creativity could yield improved acetate assimilation routes. Furthermore, regulatory engineering will likely yield strains that can seamlessly switch between substrates (e.g. co-feed acetate with minimal lag) or that are “blind” to the presence of glucose (no catabolite repression), allowing acetate to be used in combination with other carbon sources in a process. This could open up co-feeding strategies where, say, acetate and a sugar or acetate and formate are fed together to optimize yield and titer.

- Electrification and CO₂ Utilization Synergy: The integration of biomanufacturing with renewable electricity and carbon capture will deepen. Acetate sits at the intersection of these technologies, serving as an energy vector (storing electrical energy in chemical form) and a carbon vector (storing CO₂ in a microbial-ready form). We foresee improved electrochemical acetate production systems – perhaps reaching higher concentrations (>>1 M acetate) and better energy efficiency – which will make the downstream fermentation more viable. New electrode materials and reactor designs (like gas diffusion electrodes, bipolar membranes) are rapidly improving the state of CO₂ electrolysis[24]. In tandem, microbial strains will be selected or engineered to withstand the peculiarities of electrolyzer outputs (for example, high salt acetate streams). There may even be developments in bio-electro hybrid cells where microbes directly accept electrons to aid acetate assimilation. As the world moves toward decarbonization, processes that couple direct air capture of CO₂ to acetate production and then to products could become a reality. It is conceivable to imagine a future biorefinery where CO₂ from air or industry is converted to acetic acid (using solar or wind power), stored or transported as needed (acetic acid is a liquid, easier to handle than CO₂ gas), and then fermented on-site into a variety of products. This modular approach offers flexibility – one can make a stock of acetate and distribute it to different fermentation units for different products, much like crude oil is fractionated to many chemicals. The scalability of each unit operation (electrochemical and biological) will determine how soon this vision can be economically realized, but the pieces are coming together.

- New Applications and Bioprocess Designs: The versatility of acetate might spur novel applications beyond conventional fermentation. For example, bioprinting and tissue engineering could use microbial biomass grown on acetate as sustainable feed for cultured meat or microbial protein production, offering a food source decoupled from agriculture[10]. In agriculture itself, engineered microbes using acetate might be deployed for soil improvement or biocontrol, where acetate produced from crop residues in situ could feed beneficial microbes. Another emerging concept is dynamic substrate switching – using sugars for initial fast growth then switching to acetate in production phase to exploit its high yield to acetyl-CoA products. This could marry the fast kinetics of sugars with the high-yield, low-cost advantage of acetate. Continuous processes, such as chemostats or perfusion bioreactors, might also be revisited for acetate cultures, since continuous removal of acids and precise control can keep cells at optimal state. We may also see consortia where one microbe produces acetate from an upstream substrate and another consumes it to product, all in one pot (sequential mixed culture)[87] – for example, a cellulolytic microbe breaking down biomass to acetate and a production microbe converting it to chemical. Such consortia could simplify processing by avoiding complete purification of acetate.

- Policy and Economic Drivers: Finally, the trajectory will depend on external drivers. Carbon pricing, low-carbon fuel standards, or incentives for utilizing waste carbon can make acetate-based biomanufacturing more attractive economically. For instance, if CO₂-derived acetate gains a “green premium”, chemicals made from it could be marketed as climate-friendly products (with appropriate certification). This could accelerate adoption in certain sectors (e.g. biodegradable plastics made from CO₂ via acetate might fetch higher price from sustainability-minded consumers). Conversely, sustained low prices of sugar or fossil feedstocks could temper the rush to acetate unless mandated by policy or sustainability commitments. Our perspective is that global trends toward decarbonization will increasingly favor processes that incorporate CO₂ reuse. Acetate is one of the most logical intermediates for CO₂ reuse in biotechnology, so it stands to benefit from this momentum.

In conclusion, acetate has rapidly transitioned from being seen merely as a fermentation byproduct to a central feedstock candidate for next-generation biomanufacturing. Through clever metabolic engineering and process integration, many of the historical disadvantages of acetate (lower growth rate, redox demands) are being overcome, while its unique advantages (non-food origin, high carbon efficiency to acetyl-CoA, compatibility with CO₂) are being harnessed. In the next 5–10 years, we expect to witness the first commercial or near-commercial processes that prove the viability of an acetate-centric biorefinery. These could fundamentally change how we source carbon for chemicals: instead of fermenting sugars grown on arable land, we may ferment acetate made from what we throw away (waste biomass, emissions). Challenges remain, but the trajectory is clear – acetate has moved to the forefront as a linchpin feedstock linking sustainable carbon sourcing and industrial biotechnology. Its success will exemplify the power of marrying innovations in chemistry and biology to forge new paths in manufacturing. The field is at an exciting juncture, and the coming years will reveal just how far acetate can go in reshaping biomanufacturing toward a more sustainable future.

Download: This review is formatted in Markdown for convenience. Researchers may download the content and convert to PDF or Word as needed for reading or printing.

## References (APA Style)

- Wendisch, V. F., Brito, L. F., Gil Lopez, M., Hennig, G., Pfeifenschneider, J., Sgobba, E., & Veldmann, K. H. (2016). The flexible feedstock concept in industrial biotechnology: Metabolic engineering of Escherichia coli, Corynebacterium glutamicum, Pseudomonas, Bacillus and yeast strains for access to alternative carbon sources. Journal of Biotechnology, 234, 139–157. https://doi.org/10.1016/j.jbiotec.2016.07.022

- Kim, Y., Lama, S., Agrawal, D., Kumar, V., & Park, S. H. (2021). Acetate as a potential feedstock for the production of value-added chemicals: Metabolism and applications. Biotechnology Advances, 49, 107736. https://doi.org/10.1016/j.biotechadv.2021.107736

- Mutyala, S., & Kim, J. R. (2022). Recent advances and challenges in the bioconversion of acetate to value-added chemicals. Bioresource Technology, 364, 128064. https://doi.org/10.1016/j.biortech.2022.128064

- Ye, D.-K., Xie, D.-Y., Li, J.-F., Xu, P., & Liu, J.-Z. (2025). Current Status of Biological Production Using C2 Feedstocks. Synthetic Biology and Engineering, 3(2), 10007. https://doi.org/10.70322/sbe.2025.10007 (Open Access)

- Hann, E. C., Overa, S., Harland-Dunaway, M., Narvaez, A. F., Le, D. N., Orozco-Cárdenas, M. L., Jiao, F., & Jinkerson, R. E. (2022). A hybrid inorganic–biological artificial photosynthesis system for energy-efficient food production. Nature Food, 3(6), 461–471. https://doi.org/10.1038/s43016-022-00530-x

- Lee, G., Jo, H., Choi, J., Guzman, M. F., Shan, Y., Le, H. K. D., Feijoo, J., Soland, N., Clark, D. S., & Yang, P. (2025). CO₂ upgrading into bioproducts using a two-step abiotic–biotic system. Proceedings of the National Academy of Sciences USA, 122(34), e2512565122. https://doi.org/10.1073/pnas.2512565122

- Gu, P., Li, F., Huang, Z., & Gao, J. (2024). Application of acetate as a substrate for the production of value-added chemicals in Escherichia coli. Microorganisms, 12(2), 309. https://doi.org/10.3390/microorganisms12020309 (Open Access)

- Noh, M. H., Lim, H. G., Woo, S. H., Song, J., & Jung, G. Y. (2018). Production of itaconic acid from acetate by engineering acid-tolerant Escherichia coli W. Biotechnology and Bioengineering, 115(4), 729–738. https://doi.org/10.1002/bit.26510

- Kutscha, R., Tomin, T., Birner-Gruenberger, R., Bekiaris, P. S., Klamt, S., & Pflügl, S. (2024). Efficiency of acetate-based isopropanol synthesis in Escherichia coli W is controlled by ATP demand. Biotechnology for Biofuels and Bioproducts, 17, 110. https://doi.org/10.1186/s13068-024-02337-y

- Schmollack, M., Werner, F., Huber, J., Kiefer, D., Merkel, M., Hausmann, R., & Blank, L. M. (2022). Metabolic engineering of Corynebacterium glutamicum for acetate-based itaconic acid production. Biotechnology for Biofuels and Bioproducts, 15, 139. https://doi.org/10.1186/s13068-022-02178-y

- Beck, S. W., Ye, D., Hwang, H. G., & Jung, G. Y. (2024). Stepwise flux optimization for enhanced GABA production from acetate in Escherichia coli. Journal of Agricultural and Food Chemistry, 72(26), 10420–10427. https://doi.org/10.1021/acs.jafc.3c02160

- Sun, S., Ding, Y., Liu, M., Xian, M., & Zhao, G. (2020). Comparison of glucose, acetate and ethanol as carbon sources for production of poly(3-hydroxybutyrate) and other acetyl-CoA derivatives in E. coli. Frontiers in Bioengineering and Biotechnology, 8, 833. https://doi.org/10.3389/fbioe.2020.00833

- Kim, D. H., Hwang, H. G., Ye, D. Y., & Jung, G. Y. (2024). Transcriptional and translational flux optimization at the key regulatory node for enhanced production of naringenin using acetate in engineered Escherichia coli. Journal of Industrial Microbiology and Biotechnology, 51(1), kuae006. https://doi.org/10.1093/jimb/kuae006

- Seong, W., Han, G. H., Lim, H. S., Baek, J. I., Kim, S.-J., Kim, D., & Lee, S. Y. (2020). Adaptive laboratory evolution of Escherichia coli lacking cellular byproduct formation for enhanced acetate utilization through compensatory ATP consumption. Metabolic Engineering, 62, 249–259. https://doi.org/10.1016/j.ymben.2020.09.001

- Chong, H., Yeow, J., Wang, I., Song, H., & Jiang, R. (2013). Improving acetate tolerance of Escherichia coli by rewiring its global regulator cAMP receptor protein (CRP). PLoS ONE, 8(10), e77422. https://doi.org/10.1371/journal.pone.0077422

- Kamzolova, S. V., Shamin, R. V., Stepanova, N. N., Morgunov, G. I., & Finogenova, T. V. (2018). Fermentation conditions and media optimization for isocitric acid production from ethanol by Yarrowia lipolytica. Biomedical Research International, 2018, 2543210. https://doi.org/10.1155/2018/2543210

- Aryal, N., Halder, A., Zhang, M., & Tremblay, P.-L. (2023). Microbial electrosynthesis of acetate from CO₂: Current status, challenges, and perspectives. Bioresource Technology, 374, 128776. https://doi.org/10.1016/j.biortech.2023.128776

- Otte, K. B., Grunwald, L., Mahmoud, O., Jansen, T., Takors, R., & Rettberg, P. (2022). Perspectives for using CO₂ as a feedstock for biomanufacturing of chemicals and biopolymers. Engineering in Life Sciences, 22(6), 368–378. https://doi.org/10.1002/elsc.202100115

[1] [2] [4] [6] [12] [13] [14] [15] [16] [17] [21] [22] [31] [32] [33] [34] [35] [36] [37] [38] [39] [40] [41] [42] [43] [44] [45] [46] [47] [48] [49] [52] [53] [54] [55] [56] [57] [58] [59] [60] [61] [62] [63] [64] [65] [66] [67] [71] [72] [73] [74] [75] [76] [85] [86] [87] Current Status of Biological Production Using C2 Feedstocks - Synthetic Biology and Engineering - Full-Text HTML - SCIEPublish

https://www.sciepublish.com/article/pii/528

[3] [5] [9] [19] [20] [30] Application of Acetate as a Substrate for the Production of Value-Added Chemicals in Escherichia coli - PubMed

https://pubmed.ncbi.nlm.nih.gov/38399713/

[7] [10] [11] [23] [24] [25] [26] [27] [81] [82] A hybrid inorganic–biological artificial photosynthesis system for energy-efficient food production | Nature Food

https://www.nature.com/articles/s43016-022-00530-x?error=cookies_not_supported&code=8c50a8b3-954f-4a9b-979b-056aaeee60d8

[8] [83] [84] Perspectives for Using CO2 as a Feedstock for Biomanufacturing of ...

https://pmc.ncbi.nlm.nih.gov/articles/PMC10740661/

[18] [51] [68] [69] [70] Using acetate as a substrate for the production of value-added... | Download Scientific Diagram

https://www.researchgate.net/figure/Using-acetate-as-a-substrate-for-the-production-of-value-added-chemicals-in-E-coli-The_fig1_377915600

[28] [29] [80] Clark Lab - Publications

https://sites.google.com/berkeley.edu/clarklab/publications

[50] Application of Acetate as a Substrate for the Production of Value-Added Chemicals in Escherichia coli

https://www.mdpi.com/2076-2607/12/2/309

[77] Role of microbial electrosynthesis system in CO2 capture ... - Frontiers

https://www.frontiersin.org/journals/microbiology/articles/10.3389/fmicb.2023.1192187/full

[78] H2 mediated mixed culture microbial electrosynthesis for high titer ...

https://pmc.ncbi.nlm.nih.gov/articles/PMC10637882/

[79] New tech that turns CO2 into valuable chemicals, offers hope for ...

https://www.openaccessgovernment.org/new-tech-that-turns-co2-into-valuable-chemicals-offers-hope-for-sustainable-manufacturing/177767/